Platform Driver with Character Device using DTS (Compatible = "my-ldd")

1. Device Tree Node (DTS)

/ {

my_ldd_device {

compatible = "my-ldd";

status = "okay";

};

};

2. Platform Driver + Character Device Skeleton

//my_ldd_driver.c

#include <linux/module.h>

#include <linux/init.h>

#include <linux/platform_device.h>

#include <linux/of.h>

#include <linux/fs.h>

#include <linux/cdev.h>

#include <linux/device.h>

#define DRIVER_NAME "my_ldd"

#define CLASS_NAME  "my_ldd_class"

static dev_t dev_number;

static struct cdev my_cdev;

static struct class *my_class;

/* ---------------- File Operations ---------------- */

static int my_open(struct inode *inode, struct file *file)

{

printk("Device opened\n");

return 0;

}

static int my_release(struct inode *inode, struct file *file)

{

printk("Device closed\n");

return 0;

}

static ssize_t my_read(struct file *file,char __user *buf, size_t len,loff_t *off)

{

printk("Read called\n");

return 0;

}

static ssize_t my_write(struct file *file,const char __user *buf,size_t len,loff_t *off)

{

printk("Write called\n");

return len;

}

static struct file_operations fops = {

.owner   = THIS_MODULE,

.open    = my_open,

.release = my_release,

.read    = my_read,

.write   = my_write,

};

/* ---------------- Probe Function ---------------- */

static int my_probe(struct platform_device *pdev)

{

int ret;

printk("Probe function invoked (compatible = my-ldd)\n");

/* Allocate device number */

ret = alloc_chrdev_region(&dev_number,0,1,DRIVER_NAME);

if (ret < 0)

return ret;

/* Initialize cdev */

cdev_init(&my_cdev, &fops);

ret = cdev_add(&my_cdev,dev_number,1);

if (ret < 0)

goto unregister_region;

/* Create class */

my_class = class_create(THIS_MODULE, CLASS_NAME);

if (IS_ERR(my_class))

goto del_cdev;

/* Create device node */

device_create(my_class,NULL,dev_number,NULL,"my_ldd");

printk("Character device created\n");

return 0;

del_cdev:

cdev_del(&my_cdev);

unregister_region:

unregister_chrdev_region(dev_number, 1);

return -1;

}

/* ---------------- Remove Function ---------------- */

static int my_remove(struct platform_device *pdev)

{

printk("Remove function called\n");

device_destroy(my_class,dev_number);

class_destroy(my_class);

cdev_del(&my_cdev);

unregister_chrdev_region(dev_number,1);

return 0;

}

/* ---------------- Compatibility Match ---------------- */

static const struct of_device_id my_match_table[] = {

{ .compatible = "my-ldd" },

{ }

};

MODULE_DEVICE_TABLE(of, my_match_table);

/* ---------------- Platform Driver ---------------- */

static struct platform_driver my_platform_driver = {

.probe  = my_probe,

.remove = my_remove,

.driver = {

.name = "my_ldd_driver",

.of_match_table = my_match_table,

},

};

/* ---------------- Register Driver ---------------- */

module_platform_driver(my_platform_driver);

MODULE_LICENSE("GPL");

MODULE_AUTHOR("LDD Skeleton");

MODULE_DESCRIPTION("Platform Driver with Character Device and DTS");

3. Makefile

ifeq ($(KERNELRELEASE),)

KERNEL_SOURCE := /home/arun/Github/Tutorial_BeagleBoneBlack/Embitude/linux-4.19.103

PWD := $(shell pwd)

ARCH := arm

CROSS_COMPILE := arm-linux-gnueabihf-

default:

$(MAKE) -C $(KERNEL_SOURCE)	M=$(PWD) ARCH=$(ARCH) CROSS_COMPILE=$(CROSS_COMPILE) modules

clean:

$(MAKE) -C $(KERNEL_SOURCE)	M=$(PWD) ARCH=$(ARCH) CROSS_COMPILE=$(CROSS_COMPILE) clean

else

obj-m := my_ldd_driver.o

endif

If you are using BeagleBone Black (AM335x), then you must add the node into the main board DTS and recompile the device tree so the kernel can create the platform device and invoke probe().

Here is the exact workflow for your case.

Step 1️⃣ Add Node into the Board DTS

File location:

arch/arm/boot/dts/am335x-boneblack.dts

Add this node inside the root / { } section.

/ {
    my_ldd_device {
        compatible = "my-ldd";
        status = "okay";
    };
};

That is enough for compatibility matching.

Step 2️⃣ Recompile the Device Tree

From your kernel source directory:

cd linux-4.19.103

Compile the specific DTS:

make ARCH=arm CROSS_COMPILE=arm-linux-gnueabihf- \
am335x-boneblack.dtb

Or compile all DTBs:

make ARCH=arm CROSS_COMPILE=arm-linux-gnueabihf- dtbs

Output file will be:

arch/arm/boot/dts/am335x-boneblack.dtb

Step 3️⃣ Copy the New DTB to the Board

From host:

scp arch/arm/boot/dts/am335x-boneblack.dtb root@192.168.7.2:/boot/

(You already confirmed SSH/SCP connectivity in your previous step.)

Step 4️⃣ Reboot the Board

reboot

Step 5️⃣ Load Your Platform Driver

insmod my_ldd_driver.ko

Step 6️⃣ Verify probe() is Invoked

dmesg | grep Probe

Expected:

Probe function invoked (compatible = my-ldd)
Character device created

Check device:

ls /dev/my_ldd

Full Runtime Flow (Your Exact Scenario)

Edit am335x-boneblack.dts
        │
        ▼
Compile DTB
        │
        ▼
Copy DTB to /boot
        │
        ▼
Reboot BBB
        │
        ▼
insmod driver
        │
        ▼
compatible match
        │
        ▼
probe() invoked
        │
        ▼
/dev/my_ldd created

Important Notes for Your Environment (BBB + Kernel 4.19)

1️⃣ You do NOT need reg or interrupts yet

Because this is only compatibility matching.

2️⃣ Node placement matters

Correct:

/ {
    my_ldd_device {
        compatible = "my-ldd";
    };
};

Wrong:

&some_device {
    my_ldd_device { };
}

Quick Debug Commands

dmesg | grep my-ldd

ls /proc/device-tree/

ls /sys/bus/platform/devices/

You should see:

my_ldd_device

If probe() is NOT invoked, check these 5 things

- DTB copied to /boot

- Board rebooted

- compatible string matches exactly

- Driver loaded

- of_match_table_present